MNicGuidhir

2025-08-22 11:46:42

--------------------------------------------

Again, I don't know if this information is necessary. But, if you do want to include it, then verify against the information here: https://www.met.ie/forecasts/marine-inland-lakes/buoys/buoy-locations because the information here has some inaccuracies (M1 is retired, the mapping from stno-buoy name is incorrect, the descriptions of where the buoys are are incorrect, including the lat-lon locations - M5 is in Offaly according to this!!),

Met Éireann

The Irish Meteorological Service

# Storm Floris

Marine Storm Report

Marine Unit

MNicGuidhir

2025-08-22 11:42:39

--------------------------------------------

I'm not sure about including this section at all.

In the "Description" you mention widespread disruption in the West, but this was a relatively minor storm and was worst in the North.

I think the figure of 100+ km/h is incorrect - according to Sandra's table the max speed recorded on land was 80 km/h and the max buoy wind speed was lower again at around 56 km/h. For bigger storms, Climate Services Division produce a full storm report, so we can leave this sort of analysis to them.

Report Date: 20 August 2025

Report Time: 13:42 UTC

Storm Overview

Dates: 2025-08-04, 2025-08-05

Description: Unseasonably strong August Bank Holiday storm bringing widespread disruption across western Ireland.

Peak Winds: 100+ km/h

Areas Affected: West Coast, Northwest, Western counties

Marine Observations Summary

## Data Sources

- Buoy 62091 (M1 Buoy): 53.47°N, 5.42°W - West Coast

- Buoy 62092 (M2 Buoy): 53.48°N, 5.42°W - West Coast

- Buoy 62093 (M3 Buoy): 51.22°N, 6.70°W - Southwest Coast

- Buoy 62094 (M4 Buoy): 51.69°N, 6.70°W - Southwest Coast

- Buoy 62095 (M5 Buoy): 53.06°N, 7.90°W - West Coast

## Peak Conditions Observed

MNicGuidhir

2025-08-22 11:59:43

--------------------------------------------

The database windsp is in knots, so this is ~55.6km/h.

MNicGuidhir

2025-08-22 12:05:44

--------------------------------------------

18.27 I think.

MNicGuidhir

2025-08-22 12:08:25

--------------------------------------------

East

Maximum Wind Speed: 30.2 m/s (108.7 km/h) at Buoy 62093

- Maximum Significant Wave Height (Hm0): 5.4 m at Buoy 62095

MNicGuidhir

2025-08-22 12:03:46

--------------------------------------------

This is OK, but just FYI the data in the csv file still includes the 10.47m value for M5.

Maximum Wave Height (Hmax): 9.1 m at Buoy 62095

- Minimum Pressure: 1001.1 hPa at Buoy 62095

- Temperature Range: 13.5°C (Buoy 62095) to 18.7°C (Buoy 62094)

MNicGuidhir

2025-08-22 12:07:38

--------------------------------------------

There are around 500 in the csv. When backup logger is removed, this should fall further.

Total Observations: 669 records from 5 stations (QC good data only)

MNicGuidhir

2025-08-22 12:10:55

--------------------------------------------

As mentioned prev, the stno-buoy mappings, buoy lat-lon locations and region descriptions are wrong in this section.

Station-by-Station Analysis Buoy 62091 - M1 Buoy

- Location: 53.47°N, 5.42°W

- Region: West Coast

MNicGuidhir

2025-08-22 12:12:29

--------------------------------------------

Units are knots for windsp from database

Peak Wind Speed: 27.8 m/s (100.0 km/h)

### Peak Significant Wave Height (Hm0): 2.5 m

- Peak Maximum Wave Height (Hmax): 4.2 m

- Minimum Pressure: 1003.7 hPa

- Data Quality: Excellent (100.0% good data)

- Observations: 144 records (QC good data only)

## Buoy 62092 - M2 Buoy

- Location: 53.48°N, 5.42°W

- Region: West Coast

MNicGuidhir

2025-08-22 12:12:01

--------------------------------------------

should be 48 or 72 depending on how many days you include in the storm. I assume the factor of 2 is due to the dual logger. Might be better to leave out, as its a bit confusing; ex why so few records for M4 compared to the other buoys.

Peak Wind Speed: 23.3 m/s (84.1 km/h)

### Peak Significant Wave Height (Hm0): 4.1 m

- Peak Maximum Wave Height (Hmax): 8.0 m

- Minimum Pressure: 1010.7 hPa

- Data Quality: Excellent (100.0% good data)

- Observations: 153 records (QC good data only)

## Buoy 62093 - M3 Buoy

- Location: 51.22°N, 6.70°W

- Region: Southwest Coast

- Peak Wind Speed: 30.2 m/s (108.7 km/h)

### Peak Significant Wave Height (Hm0): 4.6 m

- Peak Maximum Wave Height (Hmax): 7.8 m

- Minimum Pressure: 1004.0 hPa

- Data Quality: Excellent (100.0% good data)

- Observations: 81 records (QC good data only)

## Buoy 62094 - M4 Buoy

- Location: 51.69°N, 6.70°W

- Region: Southwest Coast

- Peak Wind Speed: 29.0 m/s (104.6 km/h)

### Peak Significant Wave Height (Hm0): 3.4 m

- Peak Maximum Wave Height (Hmax): 4.4 m

MNicGuidhir

2025-08-22 12:12:38

--------------------------------------------

knots

MNicGuidhir

2025-08-22 12:15:16

--------------------------------------------

Given 30.2knots ~ 56 km/h, this probably doesn't apply. There was a small craft warning and gale warning in effect though.

MNicGuidhir

2025-08-22 12:18:31

--------------------------------------------

the "Sea State Classification" is a category assigned based on teh significant wave height only, to describe the sustained sea conditions at a location. hmax values are usually indicative of unusual waves in their context so wouldn't be relevant to the Sea State classification. Ie: it is good to include the max hmax, but don't allocate it a sea state classification because sea state and hmax are measures of different things.

Minimum Pressure: 1010.4 hPa

- Data Quality: Excellent (100.0% good data)

- Observations: 120 records (QC good data only)

## Buoy 62095 - M5 Buoy

- Location: 53.06°N, 7.90°W

- Region: West Coast

- Peak Wind Speed: 29.2 m/s (105.0 km/h)

### Peak Significant Wave Height (Hm0): 5.4 m

- Peak Maximum Wave Height (Hmax): 9.1 m

- Minimum Pressure: 1001.1 hPa

- Data Quality: Excellent (100.0% good data)

- Observations: 171 records (QC good data only)

Meteorological Analysis

## Wind Analysis

The storm produced maximum sustained winds of 30.2 m/s (108.7 km/h), representing significant marine weather conditions. Wind speeds of this magnitude pose considerable risks to marine operations and coastal areas.

### Wind Categories:

- Force 7 (Strong Gale): 13.9-17.1 m/s (50-61 km/h)

- Force 8 (Gale): 17.2-20.7 m/s (62-74 km/h)

- Force 9 (Strong Gale): 20.8-24.4 m/s (75-88 km/h)

- Force 10+ (Storm): >24.5 m/s (>88 km/h)

## Wave Analysis

Significant Wave Heights (Hm0): Peak values reached 5.4 m, representing very rough sea states according to the World Meteorological Organization classification.

Maximum Wave Heights (Hmax): Individual wave heights peaked at 9.1 m, representing very high conditions for maximum wave heights.

Wave Height Relationship: The Hmax/Hm0 ratio was 1.68, within normal range (1.3-1.8).

### Sea State Classification (Hm0):

- Rough: 2.5-4.0 m

- Very Rough: 4.0-6.0 m

- High: 6.0-9.0 m

- Very High: 9.0-14.0 m

- Phenomenal: >14.0 m

### Wave Height Definitions:

- Hm0 (Significant Wave Height): Average height of the highest one-third of waves

MNicGuidhir

2025-08-22 12:19:26

--------------------------------------------

I wouldn't include this. Particularly in this case, it's not a very notable depression. Even in more extreme cases, I would leave this kind of analysis to CSD.

MNicGuidhir

2025-08-22 12:20:26

--------------------------------------------

I would remove this section, it doesn't add anything.

Hmax (Maximum Wave Height): Highest individual wave recorded during the period

## Pressure Analysis

Atmospheric pressure dropped to a minimum of 1001.1 hPa, representing a pressure anomaly of 12.2 hPa below standard atmospheric pressure (1013.25 hPa).

### Pressure Categories:

- Normal: 1013-1023 hPa

- Low: 1000-1013 hPa

- Very Low: 980-1000 hPa

- Extremely Low: <980 hPa

Storm Timeline

Storm Period: 2025-08-03 00:00 to 2025-08-07 00:00 UTC

Duration: 4 days, 0 hours

### Key Timeline Points:

- Storm approach: Pressure began dropping and winds increased

- Peak intensity: Maximum winds and waves recorded

- Storm passage: Gradual improvement in conditions

Quality Control Summary

### Total Records: 669

QC Status Distribution:

- Good Data (QC=1): 669 records (100.0%)

- Adjusted Data (QC=5): 0 records (0.0%)

- Failed QC (QC=4): 0 records (0.0%)

- Missing Data (QC=9): 0 records (0.0%)

- No QC (QC=0): 0 records (0.0%)

Data Visualization

!Storm Overview

Figure 1: Comprehensive marine meteorological analysis showing wind speed, wave height, atmospheric pressure, air temperature, wind direction, and wave period during Storm Floris.

Technical Notes

MNicGuidhir

2025-08-22 12:21:27

--------------------------------------------

Am I right in thinking this QC'check isn't being applied yet? If not, then remove it from this list.

MNicGuidhir

2025-08-22 12:22:48

--------------------------------------------

Can I ask where you got these figures? If they are just generic estimates then I would remove them.

QC Methods Applied

- Manual QC: Visual inspection and expert validation

- Automatic QC: Range checks, spike detection, and flat-line identification

- AI-powered QC: Machine learning algorithms for anomaly detection

## Data Quality Indicators

- 0: No QC performed

- 1: QC performed, data OK

- 4: QC performed, raw data not OK and not adjusted

- 5: QC performed, raw data not OK but value adjusted/interpolated

- 6: QC performed, data OK (Datawell Hmax sensor specific)

- 9: Data missing

## Measurement Uncertainties

- Wind Speed: ±0.3 m/s

- Wave Height: ±5% or 0.5m (whichever greater)

- Atmospheric Pressure: ±0.5 hPa

- Air Temperature: ±0.2°C

---

Report generated by Marine Storm Analysis System Data source: Irish Marine Data Buoy Network Quality controlled data from Met Éireann marine observations

Marine Meteorological Analysis

MNicGuidhir

2025-08-22 12:23:51

--------------------------------------------

I think the secondary logger data is still included - just based on the slightly jagged look of the hmax and hm0 plots.

Figure 1: Marine meteorological observations during Storm Floris. Eight-panel analysis showing wind speed, significant wave height (Hm0), maximum wave height (Hmax), atmospheric pressure, air temperature, wave period, wind direction patterns, and comparative wave heights across the Irish Marine Data Buoy Network. Quality-controlled data only.

Met Éireann Marine Unit

Irish Marine Data Buoy Network

Valentia Observatory, Co. Kerry www.met.ie/climate/storm-centre